[image: image1.wmf]POEMA  SHOWER ENCLOSURE

Installation Manual

                                                                                                                     Ⅰ、Usage tool：

[image: image7.wmf][image: image8.wmf]driller        pencil         wrench 

[image: image20.wmf]   tape measure     hammer
[image: image3.wmf] 

spirit level 
[image: image4.wmf]       screwdriver  
[image: image5.wmf]      small wood block 
[image: image6.wmf]   

Ⅲ、Spare parts list： 
1.Handle 2         2.Moving glass(door) 2,    3.M5×35 stud-screw 6/ M4×35 stud-screw 8  

4. Wall plug 6,         5.Fixed glass 2，              6.M4×12From offend the screw 6，    

7 Bottom waterproof  2，      8. Bottom and upper Orbit 2,          9. Rotation base  4，      

10.Hinge  4，       11.Magnetic plastic 2,      12. Vertical waterproof  2，       

13. Sealant 2，    14.Vertical profile 2，        15.Wall profile 2, 
Ⅳ、Please check the quantity and quality of square parts before install, if you have any inquiry about it please contact our company or dealer without any hesitate, you will get satisfied reply soon!    

[image: image9.wmf]
picture 1

[image: image10][image: image11][image: image12][image: image13]Ⅴ、Installation Process：

1、 Please put the wall profile near the wall (Please

  install it according to the size of glass door),

and adjust the profile perpendicularly by spirit
  level. Then drill by Ф6—8mm driller and put the 

  wall plug into wall.
2、 Putting the vertical waterproof in side of glass 
door ,insert bottom and upper orbit and screw        picture 2
down the [image: image14.wmf]vertical profile. Then drill in both of 
frame by 3.2 driller. Screw down the decorating 
cover (picture3)by M4x12 screw and insert the 
sealant into fixed prfile by small wood board (please

       make the surface smooth).       

3、 Please install the hinges on the bottom of

moving glass door, and inset it into rotation
base. Then install the upper side same as bottom.
After that put the magnetic on the side of glass doors,

[image: image15.wmf]Please notice the magnetic pole direction. At                   

last adjust the perpendicularity for hinges by                             

wrench until the glass doors could work well.                   picture 3                 

                                                  picture 4                          

4、  After the installation, all side of glass door should be silicone.

5、  Please clear the frame by neutral purification after 8 hours.

Ⅵ、Please push the door slightly,don’t scratch the frame by strong and sharp material to avoid 
    damaging fittings.
[image: image16.wmf]
[image: image17.wmf]                       picture 5              

� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���








� EMBED AutoCAD.Drawing.16  ���





� EMBED AutoCAD.Drawing.16  ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���








[image: image18.wmf][image: image19.wmf]_1234567899.dwg

_1234567901.dwg

_1234567903.dwg

_1317467509.dwg

_1234567902.dwg

_1234567900.dwg

_1189928849.dwg

_1190630014.dwg

_1190630034.dwg

_1189942625.dwg

_1189928162.dwg

_1189928206.dwg

_1189927924.dwg

