MIA  SHOWER ENCLOSURE

Installation Manual

一、Usage tool：

driller        pencil         wrench    tape measure     hammer

spirit level        screwdriver        small wood block

二、Spare parts list:

1.ST 5×35 Stud-Screw 3 pcs/ ST 4×12 Stud-Screw 3 pcs，        2. Wall profile 1 pc，

3. Moving bar 1 pc，        4. Rotation round bar 1 pc，       5. Wall plug 3 pcs，

6. Moving glass1，       7. Waterproof 1 pc,       8.M4×30 Stud-Screw 4 pcs,

9. Bottom cover 1，      10. Bearing 2pcs       11.M4×12 2PCS         12. Cover 1 pc

三、Please check the quantity and quality of square parts before install,if you have any inquiry about it please contact our company or dealer without any hesitate, you will get satisfied reply soon!

Picture. 1

IV、Installation Process：

1. Please put the wall profile to the wall

(Accord to the wall size to install),

use the spirit level to adjust the uprightness.

After confirmation,

then use ф6—8mm driller to drill the hole,

then put the wall plug into the wall,

use M4×35 stud –screw and lock up the wall profile           Pic. 2

2. Lock up the moving bar together with the movable glass

with ST4x30 Stud-Screw, then put the movable glass

insert the wall profile( like Pic.3 )

.

3. Use ST4*12 screw to lock up the                             Pic3

inside (be sure to put the gasket on the bottom),

then put on the cover

4 Put the waterproof to the bottom of the glass, protrude inside.

5.After the installation, all side of glass door should be silicone.

Pic.4

6. Please clear the frame by neutral purification after 24 hours。

V、Please push the door slightly,don’t scratch the frame by strong and sharp material to avoid

damaging fittings.