[image: image1.wmf]VIVA  SHOWER ENCLOSURE

Installation Manual

Ⅰ、Usage tool：                                                 

[image: image7.wmf][image: image8.wmf]driller        pencil         wrench 

[image: image16.wmf]   tape measure     hammer
[image: image3.wmf] 

spirit level 
[image: image4.wmf]       screwdriver  
[image: image5.wmf]      small wood block 
[image: image6.wmf]
Ⅱ、Spare parts list：
1. Handle 1，              2. M4×35 Stud-Screw 8/M5 ×35Stud-Screw 6 pcs    

3. M4x12 stud –screw 6，    4.Wall plug 6，         5. Upper rollers 2，   

6. Upper orbit 1，           7. Bottom orbit 1，           8. Bottom bounce pulley 2，          
9. Waterproof 2，           10. Moving glass 1,          11. Sealant 1，     
12. Fixed glass1，           13. Vertical profile 1，          14. Wall profile 2.，    

15. The integral whole sealant 2

Ⅲ、Please check the quantity and quality of square parts before install, if you have any inquiry about it please contact our company or dealer without any hesitate, you will get satisfied reply soon! 

[image: image9.wmf]6

7

8

9

10

11

12

13

14

15

17

    

picture 1

[image: image10]Ⅴ、Installation Process：
[image: image11.wmf]1.  Please put the wall profile near the wall(Please install it according to the size of glass door),and adjust the profile perpendicularly by spirit level. Then drill by Ф6-8mm driller and put the wall plug into wall.

picture 2

2.  Putting the vertical waterproof in side of fixed glass, then insert it into the bottom and upper orbits and screw down the vertical profile by M4x35 screw(same as another side). Then drill in both of profiles by 3.2 driller. Then fix both profiles(picture 3) by M4x12 screw and insert the sealant into vertical profile by small wood board(please make the surface smooth). Then put the fixed glass with the vertical profile in the position of hole on wall , screw down by M4*35.

3.  Installing the upper and bottom rollers on the moving glass doors( two glass doors should be same),then insert the waterproof plastic and magnetic plastic on the side glass door and install the handle. Put the moving glass doors into orbits, Pressing the bottom rollers slightly, adjust the height of upper rollers by wrench until the glass could work well.

4.  After the installation ,all side of glass door should be silicone.

5.  Please clear the frame by neutral purification after 8 hours.                  

Ⅵ、Please push the door slightly, don’t scratch the frame by strong and sharp material to avoid damaging fittings.

[image: image12.wmf]
Picture3            

� EMBED AutoCAD.Drawing.16  ���





� EMBED AutoCAD.Drawing.16  ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���








[image: image13.wmf]6

7

8

9

10

11

12

13

14

15

17

[image: image14.wmf][image: image15.wmf]_1189928849.dwg

_1190630034.dwg

_1269408851.dwg

_1332833128.dwg

_1234567899.dwg

_1189942625.dwg

_1190630014.dwg

_1189928162.dwg

_1189928206.dwg

_1189927924.dwg

