[image: image1.wmf][image: image7]OBLIC  SHOWER ENCLOSURE

Installation Manual

                                                                                                                    Ⅰ、Usage tool：                                                 

[image: image8.wmf][image: image9.wmf]driller        pencil         wrench 

[image: image20.wmf]1

2

3

4

6

5

   tape measure     hammer
[image: image3.wmf] 

spirit level 
[image: image4.wmf]       screwdriver  
[image: image5.wmf]      small wood block 
[image: image6.wmf]  
Ⅱ、Spare parts list：
1. Bottom and upper orbit 2,         2. Fixed glass 2,       3.M4×12 Stud-Screw 6，    4. Vertical profile 2,       5.M4×35 Stud-Screw 8/M5×35 Stud-Screw6,                6. Wall profile 2,          7. Wall plug 6,        8. Waterproof 3，       9. Handle 1,        10. Moving glass(door) 1,      11. Upper and bottom rollers 4,     
12. The integral whole sealant 2.
Ⅲ、Please check the quantity and quality of square parts before install, if you have any inquiry about it please contact our company or dealer without any hesitate, you will get satisfied reply soon!    

[image: image10.wmf]1

2

3

4

6

5


picture 1

[image: image11.wmf]Ⅴ、Installation process：
1. Please put the wall profile near the wall (Please install it according to the size of fixed glass),and adjust the profile perpendicularly by spirit level. Then drill by Ф6-8mm driller and put the wall plugs into wall. At last screw down the profiles by M5x30 stud screws. 
2. Putting the waterproof in side of fixed glass, insert it into the bottom and upper orbits and screw down the vertical profile by M4x30 screw (same as another fixed glass). Then drill in both of vertical profiles by Ф3.2 driller. Fixed two vertical profiles (picture 3) by M4x12 screw and insert the sealant plastic into vertical profile by small wood board (please make the surface smooth.
3. Installing the upper and bottom rollers on the moving glass door, then insert the plastic waterproofs and magnetic plastic on the side moving glass door and install the handle. Put the moving glass door into orbits, pressing the bottom rollers slightly, adjust the height of upper rollers by wrench until the glass could work well.

4.Ater the installation, all side of glass door should be silicone.

5. Please clear the frame by neutral purification after 8 hours.                  

[image: image12.wmf][image: image13.wmf]
                 picture 2                           picture 3
[image: image14.wmf]Ⅵ、Please push the door slightly,don’t scratch the frame by strong and sharp material to avoid 
damaging fittings.
picture 4                       

� EMBED  \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16  ���





� EMBED AutoCAD.Drawing.16  ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���








[image: image15.wmf] 

[image: image16.wmf][image: image17.wmf][image: image18.wmf][image: image19.wmf]_1189928849.dwg

_1234567890.doc
[image: image1.png]DOLLOR 2%

Dotlor








_1317454867.dwg

_1384606318.dwg

_1234567899.dwg

_1190630014.dwg

_1190630034.dwg

_1189942625.dwg

_1189928162.dwg

_1189928206.dwg

_1094275609.dwg

_1189927924.dwg

_1090216671.dwg

