[image: image1.wmf]MOTION SHOWER ENCLOSURE

Installation Manual
Ⅰ、Usage tool：                                                    

driller 

[image: image16.wmf]    pencil      wrench 
[image: image3.wmf]      tapeline  
[image: image4.wmf]     hammer 
[image: image5.wmf]   

level rule 
[image: image6.wmf]       screwdriver  
[image: image7.wmf]   small wood block 
[image: image8.wmf]  

Ⅱ、Spare parts list：

1. Handle 1set,   
2. M4×30 Stud-screw 20pcs,      
3. Inflate tube 12pcs,       
4. Movable glass 1pc,   

5. M4×12 Stud-screw 6pcs， 
6. Magnetic stripe 1pc.  
7. Upper orbit 1pcs (L/R)   
8. Bottom orbit 1pcs (L/R),   

9. Slide pulley 4pcs,    
10. Fixed glass 1pc,     
11. Plane magnetic stripe 1pc,   
12. Fixed frame 2pcs,    
13. Wall frame 2pcs,    
14. The single side airproof seal 1pcs       
[image: image9.wmf]15. Decorate cover for wall frame 2pcs
Picture1

[image: image10.wmf]Ⅲ、Please check the quantity and quality of square parts before install, if you have any inquiry about it please contact our company or dealer without any hesitate, you will get satisfied reply soon!

Ⅳ、Installation process：
[image: image11.wmf]11

1. Please measure the size on the wall and adjust the perpendicular degree according to the lever ruler, then draw the hole by ф6-8mm driller and put the inflate tube into wall. .

[image: image12.wmf]
Picture2                               picture3

2. Putting the plastic in both ends of the fixed glass, inset bottom and upper orbit and screw down the fixed frame. Put the wall frame into the appointed position of the fixed frame and screw down it by M4x30 stud-screw, Then drill in both of frame by 3.2 driller. Screw down it by M4x12 stud-screw and insert the airproof plastic into fixed frame by small wood board.

3. Installing the slide pulley on the movable glass door (two glass door should be symmetrical), then inset the waterproof plastic and magnetism piece on the side glass door(magnetism piece should be at handle’s side) and put the moved glass door into orbit, press the bottom slide pulley slightly, adjust the height of slide pulley by wrench until the glass could work well. Put the decorate cover for wall frame. 

4. After the installation, all side of glass door should be sealed by glass glue.

5. Wash the frame by neuter cleanser after 8 hours.
[image: image13.wmf]Ⅴ、Please push the door slightly,don’t scratch the frame by strong and sharp material to avoid fittings be damaged.
Picture 4
� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���





� EMBED AutoCAD.Drawing.16  ���





� EMBED AutoCAD.Drawing.16 \* MERGEFORMAT ���








[image: image14.wmf][image: image15.wmf]11

_1234567893.dwg

_1234567895.dwg

_1234567897.dwg

_1404285038.dwg

_1234567896.dwg

_1234567894.dwg

_1234567891.dwg

_1234567892.dwg

_1077881550.dwg

_1234567890.dwg

_1077882114.dwg

_1077881267.dwg

